Project Technical Manual & Thesis Blueprint: Auto Lighting System

1. Project Overview & Comprehensive Details

The Auto Lighting System is an ambient-responsive, intelligent illumination ecosystem designed to transition residential and urban environments from passive hardware to autonomous IoT networks. By continuous real-time monitoring of natural light levels and human occupancy patterns, the system automatically calibrates brightness and color temperature without human intervention.

- Problem Statement: Traditional lighting infrastructure relies heavily on manual control, resulting in static operations that cannot adapt to natural light availability or vacancy. This causes severe energy wastage (accounting for a 40% to 60% inflation in power bills), biological disruptions to human circadian rhythms, and accelerates global carbon emissions.

- Core Objective: To design and prototype a wireless, modular smart lighting system that automatically cuts energy overheads by 40–60%, improves indoor task safety, and aligns explicitly with United Nations Sustainable Development Goals: SDG 7 (Affordable & Clean Energy), SDG 11 (Sustainable Cities), and SDG 13 (Climate Action).

2. Technical Design & Architecture Approach

To maximize efficiency, stability, and ease of deployment, the system employs a low-latency Edge-AI IoT architecture. This avoids massive retrofitting and rewiring costs by leveraging wireless network dominance.

System Architecture Flow

Sensors (LDR + PIR)⟶Edge Microcontroller (ESP32 Node)⟶PWM Controller⟶Dimming LED Array

1. Data Acquisition Layer (Sensors)

- Photoelectric Sensor (LDR): Tracks changing ambient daylight intensities throughout the day.

- Passive Infrared Sensor (PIR): Detects human motion and occupancy inside target zones.

2. Processing & Edge-AI Layer

- The data from the sensors is processed locally by a microcontroller acting as an Edge AI Processing Unit.

- The Predictive Algorithm: Instead of rigid threshold switching, the edge processor utilizes a localized polynomial smoothing equation to map ambient lux (L) to the required Pulse Width Modulation duty cycle (D).

- To prevent sudden, jarring light flickers when a cloud passes by, a simple moving average filter is applied to the sensor inputs:

Dtarget=max0,min100,α⋅Ltarget-Lambient

3. Control & Output Layer

- PWM Dimming: The micro-controller generates Pulse-Width Modulation signals to cycle power to the driver at high frequencies, achieving precise control from 1% to 100% brightness.

- Wireless Dashboard Integration: Operates on local Wi-Fi or Bluetooth protocols to send real-time consumption feedback to a unified monitoring dashboard.

3. Mechanical Approach & Component Matrix

The structural breakdown of hardware required to physically construct the prototype:

| Component Category | Part Name / Model | Quantity | Technical Purpose / Connection Role |
| --- | --- | --- | --- |
| Main Processing Unit | ESP32 Wi-Fi + Bluetooth MCU | 1 | Acts as the AI Processing Unit; reads sensor data and outputs wireless metrics and PWM signals. |
| Light Sensing | Photoresistor (LDR) / BH1750 | 1 | Continuously measures ambient light levels (Lux). Connected via an analog pin or I2C. |
| Occupancy Detection | HC-SR501 PIR Motion Sensor | 1 | Detects human movement across a 120° cone. Connected to a digital input pin. |
| Dimming Mechanism | IRF540N MOSFET Module / Optocoupler | 1 | Acts as the PWM controller hardware. Translates low-voltage logic into high-current power gating. |
| Illumination Source | 12V DC LED Strip or Intelligent LED Bulb | 1 Block | The primary light source capable of analog or PWM intensity scaling. |
| System Power | 12V 2A DC Power Adapter + Step-Down | 1 | Provides high-power rail for LEDs and steps down to 5V/3.3V for the ESP32. |

4. Virtual Simulation Platforms (Online Prototyping)

To present a virtual interactive proof-of-concept before purchasing real hardware, use these online assembly simulators:

1. Wokwi (Recommended for ESP32 and Advanced IoT)

- What it does: An online electronic simulator that perfectly compiles Arduino, ESP32, and Raspberry Pi Pi Pico projects in real time. It natively supports PIR sensors, LDRs, and PWM-controlled LED segments.

- Why it's perfect for this: You can write actual C++/Python code, see the virtual LED dim smoothly depending on your slider inputs for light, and simulate data transmission over a virtual internet connection.

2. Autodesk Tinkercad Circuits

- What it does: A highly visual, drag-and-drop wiring simulator.

- Why it's perfect for this: Great for initial structural and mechanical diagram proofs. You can visually wire a microcontroller, plug in an adjustable light sensor (LDR), and observe oscilloscope waves changing through PWM.

5. Detailed Connection Process & Step-by-Step Wiring Manual

Follow these step-by-step instructions to map out your virtual wiring on Wokwi/Tinkercad or to assemble your real physical component prototype.

Step 1: Wiring the LDR (Ambient Light Monitor)

- Place the LDR on a breadboard. Connect one terminal to the 3.3V pin of the ESP32.

- Connect the second terminal to an analog input pin (e.g., GPIO 4 / ADC1).

- Connect a 10kΩ pull-down resistor from that same analog pin terminal to the GND pin. This creates a voltage divider network to translate light resistance variations into readable analog voltages.

Step 2: Wiring the PIR Sensor (Occupancy Core)

- Connect the VCC pin of the PIR sensor to the 5V / VIN pin of the micro-controller board.

- Connect the GND pin of the PIR sensor to the common GND rail on your layout.

- Connect the middle OUT digital pin of the PIR to GPIO 19.

Step 3: Wiring the PWM Controller & Output Array

Safety Warning: Do not connect high-draw LED arrays directly to microcontroller GPIO pins to prevent thermal damage. Always route power through a logic-level switching MOSFET.

- Connect GPIO 18 (designated as your PWM output) to the Gate (G) pin of the IRF540N MOSFET module.

- Connect the Source (S) pin of the MOSFET directly to the common GND rail.

- Connect the negative terminal (-) of your 12V LED Strip to the Drain (D) pin of the MOSFET.

- Connect the positive terminal (+) of your 12V LED Strip directly to the positive terminal of the 12V External Power Supply.

- Connect the negative ground terminal of the 12V external power supply directly to the micro-controller's GND pin to establish a unified reference ground.

Step 4: Logic Configuration & Testing Loop

- State A (Space is Empty): If PIR input reads LOW, the PWM signal writes a duty cycle value of 0 (system overrides to an off state to save 100% of passive overheads).

- State B (Space is Occupied): When PIR goes HIGH, the AI loop reads the current ADC value from the LDR. If daylight is abundant, the PWM duty cycle hovers low (1% to 20%). As natural light diminishes, the edge logic linearly scales the PWM value upward to 100% intensity, matching standard task constraints flawlessly.